Supplementary Roughness Equations - Vegetation Roughness of Trunks, Branches and Leaves

1. Rigid Trunk Roughness

This supplementary section describes approaches to calculate vegetative roughness of trunks, branched and leaved elements using the Darcy-Weisbach equations. Darcy-Weisbach’s roughness values are all translated to Manning’s n. Manning’s n is the most widely used factor in hydraulic models, where guideline tables have been developed for decades relating Manning’s roughness coefficient to various boundary characteristics (Chow, 1959; USGS, 1989, [1,2]). Much of the recent work on parameterising friction of rigid and flexible vegetation elements has used the Darcy-Weisbach equation (e.g. Fathi-Moghadam & Kouwen 1997; Mason et al. 2003; Järvelä et al. 2004; Västilä & Järvelä 2017, [3–6]), however the two coefficients are interrelated mathematically. The friction factor for flow resistance caused by vegetation roughness can be defined as (Järvelä, 2004, [5]):

| f=4CdAPhAB | (S1) |
| --- | --- |

Where the friction factor (f) is related to the drag coefficient (Cd), the projected area of the plant blockage (AP) with increase in flow depth (h), and the bottom horizontal area occupied by the plant (AB). Darcy-Weisbach’s friction factor f can be converted to Manning’s n (Fathi-Moghadam & Kouwen 1997, [3]) as:

| n=f h1/38g | (S2) |
| --- | --- |

For rigid cylinder roughness such as tree trunks, the projected plant area index (AP/AB) stated in Equation 1 can be related to the average diameter of the cylindrical trunks (d) with flow depth (h), divided by the longitudinal (ax) and the lateral (ay) distances between the elements. Therefore, in these cases (Järvelä, 2004; Lindner, 1982; Antonarakis et al. 2008, [3,7,8] ):

| APhAB=dhaxay=SAI(h) | (S3) |
| --- | --- |

The stem area index (SAI) is the one-sided area sum of all stems per plot or horizontal area. Lindner (1982) [7] derived an empirical formula of the stem drag coefficient (Cd,S) for simple cylindrical vegetated stands:

| Cd,S=1+1.9dayCd∞0.2025axd0.46Cd∞+2ayay-d-2 | (S4) |
| --- | --- |

Cd∞is the drag coefficient of a single cylinder in an ideal 2D flow and is reported to be 1.0-1.2 for typical Reynolds numbers. The two terms on the right-hand side of the above equation represent blockage and free surface effects respectively.

2. Flexible Leafless Roughness

Floodwater entering the forest canopy comes into contact with its stem, as well as its branches and leaves. This is important not only for more severe floods where the water level could be high, but also for natural riparian forests where the trunk is short and the canopy layer begins relatively near the ground. Recent studies have determined and derived leafless roughness to be stiff (e.g. Pasche & Rouvé, 1985; Antonarakis et al. 2010, [9,10]). Yet, even for partially submerged vegetation, branches and twigs will bend under pressure from flow. For flexible flow, Fathi-Moghadam (1996) [11] and Järvelä (2004) [5] defined a dimensionless vegetation parameter α, accounting for the deformation of plants in flow, related to velocity α=U/Uxχ. Parameter χ describes the relationship between the species-specific drag force and flow velocity resulting from flexibility induced reconfiguration. For rigid elements χ = 0, for flexible leafless conditions χ = −0.2…−0.5, and for flexible foliated conditions χ = −0.4…−1.0 (Västilä & Järvelä 2017, [6]). Conifer foliated deformations are lower than deciduous trees, e.g. χ = −0.4…−0.55 and χ = −0.6…−1.0 (Aberle & Järvelä 2013, [12]). Uχ is the lowest velocity used in determining χ and is typically 0.1-0.2 m/s, and U is mean cross-sectional velocity.

Furthermore, the full woody friction factor of a location or pixel (xy) is the aggregate of friction calculated for all trees of sizes (z) within a species (i). This consideration is especially important in real forested plots of mixed species and sizes. Therefore the frontal area vertical profile of leafless elements per bottom area can be considered the same as the Wood Area Index (WAI) vertical profile:

| WAIih,xy=1zAWizh,xyABxy | (S5) |
| --- | --- |

Where W = woody, and considers the frontal projected area of woody branches and twigs. Each individual tree (z) in Equation 5 within a plot (xy) is associated with its own frontal area belonging to the same plot area (AB). If measuring the WAI of a single tree where the plot area has not been defined, the bottom area is the horizontal (2D) crown area.

If the deformation factor and tree species and size information is included in Equation 1, the leafless/woody (W) Darcy-Weisbach friction factor becomes:

| fWh,xy=i4Cdi,WUUχi,wχi,wWAIih,xy | (S6) |
| --- | --- |

The above equation has another aggregation, now for species (i). Here, the total friction with flow depth (h) is calculated as the sum of all trees of all species within a plot. Note that i in Equations 5 & 6 can also denote plant functional types, or a combination of tree species with similar functional traits – i.e. plants sharing similar biophysical, physiological, and morphological properties.

The species-specific drag coefficient of the woody Cdi,W in equation 6 is not the same as the trunk drag coefficient of rigid elements as in Equation 4. Values for the drag coefficient of branches and leafless trees have been estimated between Cd = 1–1.5 (Järvelä 2002; DVWK, 1991, [13,14]). Yet these values may diverge when considering complex vegetation structures of very tall trees, flexible branches, and different species. Recent studies by Västilä and Järvelä (2014) [15], and Västilä and Järvelä (2017) [6] have defined the woody drag coefficient for various hardwood and softwood species between 0.84–1.03.

3. Flexible Leafy Roughness

The presence of leaves in the form of leaf area and leaf density, can significantly increase vegetation roughness. The flexibility of foliage elements in flow is very important to consider in estimated roughness, as the total surface area occupied by leaves is usually multiple times higher than the projected area of branches, trunks and twigs. Therefore, similarly to Equation 6, the flow resistance equation for flexible foliated elements can be described as:

| fFh,xy=i4Cdi,FUUχi,fχi,fLAIih,xy | (S7) |
| --- | --- |

The one-sided Leaf Area Index with flow depth (LAIih,xy) is the non-linear vertical foliage profile with height for each species i. The species-specific drag coefficient of foliage elements (Cdi,F) has recently been defined by Aberle & Järvelä (2013), Västilä and Järvelä (2014), and Västilä and Järvelä (2017) [6,12,15] for various hardwood and softwood species between 0.09–0.69. Species specific drag coefficients have also been estimated for various trees in other studies e.g. Fathi-Moghadam (1996), Kane & Smiley (2006), Wurder et al. (2011) [11,16,17]. Recently, vegetation parameter χf has been determined for flexible foliage as well as woody components, and is given in Järvelä (2004), Västilä & Järvelä (2017), Aberle & Järvelä (2013) [5,6,12].

References:

- Chow. V.Y. Open Channel Hydraulics. McGraw-Hill: New York, USA, 1959.

- USGS, Guide for Selecting Manning’s Roughness Coefficients for Natural Channels and Flood Plains. Water Supply Paper 2339, USGS: Denver, USA, 1989.

- Fathi-Moghadam, M., Kouwen, N. Nonrigid, nonsubmerged, vegetative roughness on floodplains. Journal of Hydraulic Engineering 1997, 123 (1), 51–57.

- Mason, D.C., Cobby, D.M., Horritt, M.S., Bates, P.D. Floodplain friction parameterization in two-dimensional river flood models using vegetation heights derived from airborne scanning laser altimetry. Hydrol. Process. 2003, 17, 1711–1732.

- Järvelä, J. Determination of flow resistance caused by non‐submerged woody vegetation. International Journal of River Basin Management 2004, 2(1), 61-70.

- Västilä, K. and Järvelä, J., Characterizing natural riparian vegetation for modeling of flow and suspended sediment transport. J. Soils Sediments 2017, 3, 1–17.

- Lindner, K. Der Strömungswiderstand von Pflanzenbeständen. Mitteilungen 75. Doctoral Thesis, Leichtweiss-Institut für Wasserbau, TU Braunschweig, German, 1982.

- Antonarakis, A.S., Richards, K.S., Brasington, J., Bithell, M. and Muller, E. Retrieval of vegetative fluid resistance terms for rigid stems using airborne lidar. Journal of Geophysical Research: Biogeosciences 2008 113. DOI:10.1029/2007JG000543

- Pasche, E., Rouvé, G. Overbank flow with vegetatively roughened flood plains. Journal of Hydraulic Engineering 1985, 111(9), 1262–1278.

- Antonarakis, A.S., Richards, K.S., Brasington, J. and Muller, E. Determining leaf area index and leafy tree roughness using terrestrial laser scanning. Water Resources Research 2010, 46. DOI: 10.1029/2009WR008318

- Fathi-Moghadam, M. Momentum absorption in non-rigid, non-submerged, tall vegetation along rivers. Doctoral thesis, University of Waterloo, Waterloo, Canada, 1996.

- Aberle, J. and Järvelä, J. Flow resistance of emergent rigid and flexible floodplain vegetation. J. Hydraul. Res. 2013, 51, pp.33–45.

- Järvelä, J. Flow resistance of flexible and stiff vegetation: a flume study with natural plants. J. Hydrol. 2002, 269, 44–54.

- DVWK Hydraulische Berechnung von Fließgewässern. Deutscher Verband für Wasserwirtschaft und Kulturbau e.V.: Bonn, German, 1991.

- Västilä, K. and Järvelä, J., Modeling the flow resistance of woody vegetation using physically based properties of the foliage and stem. Water Resour. Res. 2014, 50(1), 229-245.

- Kane, B., Smiley, E.T. Drag coefficient and crown area estimation of red maple. Can. J. For. Res. 2006, 36, 1951–1958.

- Wunder, S., Lehmann, B. and Nestmann, F. Determination of the drag coefficients of emergent and just submerged willows. Int. J. River Basin Manag. 2011, 9(3-4), pp.231-236.